Catchment boundaries, daily and sub-daily hydrometeorological time series, groundwater level time series and attributes for 671 catchments in Great Britain (CAMELS-GB v2)

Summary

This dataset provides catchment boundaries, hydro-meteorological timeseries and landscape attributes for 671 catchments across Great Britain.

It collates river flows, river levels, groundwater levels, precipitation, potential evapotranspiration and temperature time series at monthly to hourly timescales. Daily hydro-meteorological timeseries are provided from 1st October 1970 – 30th September 2022, hourly hydro-meteorological timeseries are provided from 1st October 1990 09:00 to 1st October 2022 08:00, and groundwater level timeseries cover a range of time periods (ranging from 7 – 72 years with the earliest records beginning in the 1950s).

A comprehensive set of catchment attributes are quantified describing a range of catchment characteristics including topography, climate, hydrology, land cover, soils, hydrogeology, hydrometry and human influences.

We recommend that users read the accompanying research paper (Coxon et al, CAMELS-GB v2: Hydrometeorological time series and landscape attributes for 671 catchments in Great Britain) in Earth System Science Data (https://www.earth-system-science-data.net/) which describes the dataset in detail.

Updates

In response to reviewer comments on the accompanying CAMELS-GB v2 data paper in Earth System Science Datasets (https://essd.copernicus.org/preprints/essd-2025-608/), we have made some minor changes to the dataset. The updated dataset includes new data in two of the attribute files and changes how missing values are represented in one of the attribute files so it is consistent across the dataset.  The changes affect three files:

CAMELS_GB_v2_climatic_attributes.csv – NA values are changed to NaN so they are consistent with the rest of the dataset.

CAMELS_GB_v2_groundwaterwell_attributes.csv – two new columns of data have been added describing the latitude and longitude of each groundwater well.

CAMELS_GB_v2_ hydrometry_attributes.csv – four new columns of data have been added describing the quality of the flow data collected at each station (station_quality_pooling, station_quality_qmed, station_quality_hourlyflow_issues, station_quality_hourlyflow_comment).

Data Structure and File Formats

There are three primary folders:

- Catchment_Attributes

This folder includes 9 csv files containing data on the catchment attributes for different attribute classes. More information on the data within these files can be found in Table 1.

CAMELS_GB_v2_climatic_attributes.csv

CAMELS_GB_v2_groundwaterwell_attributes.csv

CAMELS_GB_v2_humaninfluence_attributes.csv

CAMELS_GB_v2_hydrogeology_attributes.csv

CAMELS_GB_v2_hydrologic_attributes.csv

CAMELS_GB_v2_hydrometry_attributes.csv

CAMELS_GB_v2_landcover_attributes.csv

CAMELS_GB_v2_soil_attributes.csv

CAMELS_GB_v2_topographic_attributes.csv

- Catchment_Boundaries

This folder includes 7 files containing the shapefile of the catchment boundaries on OSGB projection.

CAMELS_GB_v2_catchment_boundaries.dbf

CAMELS_GB_v2_catchment_boundaries.prj

CAMELS_GB_v2_catchment_boundaries.sbn

CAMELS_GB_v2_catchment_boundaries.sbx

CAMELS_GB_v2_catchment_boundaries.shp

CAMELS_GB_v2_catchment_boundaries.shp.xml

CAMELS_GB_v2_catchment_boundaries.shx

- Catchment_Timeseries

This folder has two sub-folders; groundwater and hydro-meteorological timeseries. Each of these folders also contains two sub-folders depending on the temporal resolution of the timeseries (monthly, daily or hourly). All dates in the timeseries files are provided as YYYY-MM-DD. More information on the data within these files can be found in Tables 2-4.

Groundwater > Daily

This folder includes 23 csv files containing data on the daily groundwater level timeseries with the following naming convention:

CAMELS_GB_v2_groundwater_daily_timeseries_StationID_YYYYMMDD-YYYYMMDD.csv

Where YYYYMMDD-YYYYMMDD refers to the start and end date the measurements were recorded and StationID is the groundwater well identifier at which the measurements were recorded.

Groundwater > Monthly

This folder includes 56 csv files containing data on the monthly groundwater level timeseries with the following naming convention:

CAMELS_GB_v2_groundwater_monthly_timeseries_StationID_YYYYMMDD-YYYYMMDD.csv

Where YYYYMMDD- YYYYMMDD refers to the start and end date the measurements were recorded and StationID is the groundwater well identifier at which the measurements were recorded.

Hydro-Meteorological > Daily

This folder includes 671 csv files containing data on the daily hydro-meteorological timeseries with the following naming convention:

CAMELS_GB_v2_hydromet_daily_timeseries_ StationID_YYYYMMDD-YYYYMMDD.csv

Where YYYYMMDD- YYYYMMDD refers to the start and end date the measurements were recorded and StationID is the catchment identifier (corresponds to the gauging station ID provided by the National River Flow Archive) at which the measurements were recorded.

Hydro-Meteorological > Hourly

This folder includes 671 csv files containing data on the hourly hydro-meteorological timeseries with the following naming convention:

CAMELS_GB_v2_hydromet_hourly_timeseries_ StationID_YYYYMMDD-YYYYMMDD.csv

Where YYYYMMDD- YYYYMMDD refers to the start and end date the measurements were recorded and StationID is the catchment identifier (corresponds to the gauging station ID provided by the National River Flow Archive) at which the measurements were recorded.

Nature and units of recorded values

Tables 1-4 describe the variables, their units and underlying data sources of the data provided in the catchment attribute and timeseries files. All missing values are indicated by ‘NaN’.

Table 1. Summary table of attributes available in CAMELS-GB v2

| Attribute Class | Attribute Name | Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Location and Topography | gauge_id | catchment identifier (corresponds to the gauging station ID provided by the NRFA) | - | UK National River Flow Archive using the NRFA API* |
|  | gauge_name | gauge name (river name followed by gauging station name) | - |  |
|  | gauge_lat | gauge latitude | ° |  |
|  | gauge_lon | gauge longitude | ° |  |
|  | gauge_easting | gauge easting | m |  |
|  | gauge_northing | gauge northing | m |  |
|  | gauge_elev | gauge elevation | m.a.s.l |  |
|  | area | catchment area | km2 | CEH’s Integrated Hydrological Digital Terrain Model (Morris and Flavin, 1990) |
|  | dpsbar | catchment mean drainage path slope | m km-1 |  |
|  | elev_mean | catchment mean elevation | m.a.s.l |  |
|  | elev_min | catchment minimum elevation | m.a.s.l |  |
|  | elev_10 | catchment 10th percentile elevation | m.a.s.l |  |
|  | elev_50 | catchment median elevation | m.a.s.l |  |
|  | elev_90 | catchment 90th percentile elevation | m.a.s.l |  |
|  | elev_max | catchment maximum elevation | m.a.s.l |  |
| Climatic Indices (computed for 1st Oct 1970 to 30th Sept 2022) | p_mean | mean daily precipitation | mm day-1 | Catchment timeseries of precipitation, potential evapotranspiration and temperature described in Table 2 |
|  | pet_mean | mean daily PET (Penman-Monteith equation without interception correction) | mm day-1 |  |
|  | aridity | aridity, calculated as the ratio of mean daily potential evapotranspiration to mean daily precipitation | - |  |
|  | p_seasonality | seasonality and timing of precipitation (estimated using sine curves to represent the annual temperature and precipitation cycles; positive (negative) values indicate that precipitation peaks in summer (winter) and values close to zero indicate uniform precipitation throughout the year) | - |  |
|  | frac_snow | fraction of precipitation falling as snow (for days colder than 0°C) | - |  |
|  | high_prec_freq | frequency of high precipitation days (≥ 5 times mean daily precipitation) | days yr-1 |  |
|  | high_prec_dur | average duration of high precipitation events (number of consecutive days ≥ 5 times mean daily precipitation) | days |  |
|  | high_prec_timing | season during which most high precipitation days (≥ 5 times mean daily precipitation) occur. If two seasons register the same number of events, a value of NaN is given. | season |  |
|  | low_prec_freq | frequency of dry days (< 1mm day-1) | days yr-1 |  |
|  | low_prec_dur | average duration of dry periods (number of consecutive days < 1mm day-1) | days |  |
|  | low_prec_timing | season during which most dry days (< 1mm day-1) occur. If two seasons register the same number of events, a value of NaN is given. | season |  |
| Hydrologic Signatures (computed for 1st Oct 1970 to 30th Sept 2022) | q_mean | mean daily discharge | mm day-1 | Catchment timeseries of streamflow and precipitation described in Table 2 |
|  | runoff_ratio | runoff ratio, calculated as the ratio of mean daily discharge to mean daily precipitation | - |  |
|  | stream_elas | streamflow precipitation elasticity (sensitivity of streamflow to changes in precipitation at the annual timescale, using the mean daily discharge as reference) | - |  |
|  | slope_fdc | slope of the flow duration curve (between the log-transformed 33rd and 66th streamflow percentiles) | - |  |
|  | baseflow_index | baseflow index (ratio of mean daily baseflow to daily discharge, hydrograph separation performed using the Ladson et al., 2013 digital filter) | - |  |
|  | baseflow_index_ceh | baseflow index (ratio of mean daily baseflow to daily discharge, hydrograph separation performed using the Gustard et al., 1992 method) | - |  |
|  | hfd_mean | mean half-flow date (date on which the cumulative discharge since 1 October reaches half of the annual discharge) | days since 1st October |  |
|  | Q5 | 5% flow quantile (low flow) | mm day-1 |  |
|  | Q95 | 95% flow quantile (high flow) | mm day-1 |  |
|  | high_q_freq | frequency of high-flow days (> 9 times the median daily flow) | days yr-1 |  |
|  | high_q_dur | average duration of high flow events (number of consecutive days >9 times the median daily flow) | days |  |
|  | low_q_freq | frequency of low flow days (< 0.2 times the mean daily flow) | days yr-1 |  |
|  | low_q_dur | average duration of low flow events (number of consecutive days < 0.2 times the mean daily flow) | days |  |
|  | zero_q_freq | frequency of days with Q = 0 | % |  |
| Land Cover Attributes | dwood_perc_YYYY | percentage cover of deciduous woodland for that year (YYYY) | % | 25 m raster data, Land Cover Map 1990, 2015, 2017, 2018, 2019, 2020, 2021, 2022 (Marston et al., 2022, 2024; Morton et al., 2020a, b, c, 2021; Rowland et al., 2017, 2020) |
|  | ewood_perc_YYYY | percentage cover of evergreen woodland for that year (YYYY) | % |  |
|  | grass_perc_YYYY | percentage cover of grass and pasture for that year (YYYY) | % |  |
|  | shrub_perc_YYYY | percentage cover of medium scale vegetation (shrubs) for that year (YYYY) | % |  |
|  | crop_perc_YYYY | percentage cover of crops for that year (YYYY) | % |  |
|  | urban_perc_YYYY | percentage cover of suburban and urban for that year (YYYY) | % |  |
|  | inwater_perc_YYYY | percentage cover of inland water for that year (YYYY) | % |  |
|  | bares_perc_YYYY | percentage cover of bare soil and rocks for that year (YYYY) | % |  |
| Soil Attributes Each soil attribute is accompanied by the percentage missing and the 5th, 50th and 95th percentile (apart from percentage sand, silt, clay and organic content) of that attribute across the catchment | sand_perc | percentage sand | % | European Soil Database Derived Data product (Hiederer, 2013a, b), and the modelled depth to bedrock global product (Pelletier et al., 2016) |
|  | silt_perc | percentage silt | % |  |
|  | clay_perc | percentage clay | % |  |
|  | organic_perc | percentage organic content | % |  |
|  | bulkdens | bulk density | g cm-3 |  |
|  | tawc | total available water content | mm |  |
|  | porosity_cosby | volumetric porosity (saturated water content estimated using a pedotransfer function based on sand and clay fractions) | - |  |
|  | porosity_hypres | volumetric porosity (saturated water content estimated using a pedotransfer function based on silt, clay and organic fractions, bulk density and topsoil) | - |  |
|  | conductivity_cosby | saturated hydraulic conductivity (estimated using a pedotransfer function based on sand and clay fractions) | cm h-1 |  |
|  | conductivity_hypres | saturated hydraulic conductivity (estimated using a pedotransfer function based on silt, clay and organic fractions, bulk density and topsoil) | cm h-1 |  |
|  | root_depth | depth available for roots | m |  |
|  | soil_depth_pelletier | depth to bedrock (maximum 50m) | m |  |
| Hydrogeology Attributes | inter_high_perc | significant intergranular flow – high productivity | % | British Geological Survey hydrogeology map (BGS hydrogeology 625k, 2019) and superficial deposits map |
|  | inter_mod_perc | significant intergranular flow – moderate productivity | % |  |
|  | inter_low_perc | significant intergranular flow – low productivity | % |  |
|  | frac_high_perc | flow through fractures – high productivity | % |  |
|  | frac_mod_perc | flow through fractures – moderate productivity | % |  |
|  | frac_low_perc | flow through fractures – low productivity | % |  |
|  | no_gw_perc | rocks with essentially no groundwater | % |  |
|  | low_nsig_perc | generally low productivity (intergranular flow) but some not significant aquifer | % |  |
|  | nsig_low_perc | generally not significant aquifer but some low productivity (intergranular flow) | % |  |
| Hydrometry | station_type | gauging station type denoted by the following abbreviations (B Broad-crested weir; C Crump profile single-crest weir; CB Compound broad-crested weir; CC Compound Crump weir; EM Electromagnetic gauging station; EW Essex weir; FL Flume; FV Flat V triangular profile weir; IV Index Velocity; MIS Miscellaneous; TP Rectangular thin-plate weir; US Ultrasonic gauging station; VA Velocity-area gauging station; VN Triangular (V notch) thin-plate weir); XC Cross Correlation. Two abbreviations may be applied to each station relating to the measurement of low or high flows. | - | UK National River Flow Archive using the NRFA API* |
|  | daily_flow_period_start | first date that daily flow time series provided in CAMELS-GB v2 is available for this gauging station | - | Catchment timeseries of daily streamflow described in Table 2 |
|  | daily_flow_period_end | end date that daily flow time series provided in CAMELS-GB v2 are available for this gauging station | - |  |
|  | daily_flow_perc_complete | percentage of days with flow time series available from 1st October 1970 – 31st September 2022 | % |  |
|  | hourly_flow_period_start | first date that hourly flow time series provided in CAMELS-GB v2 is available for this gauging station | - | Catchment timeseries of hourly streamflow described in Table 3 |
|  | hourly_flow_period_end | end date that hourly flow time series provided in CAMELS-GB v2 are available for this gauging station | - |  |
|  | hourly_flow_perc_complete | percentage of hours with flow time series available from 1st October 1990 09:00:00 UTC – 1st October 2022 08:00:00 UTC | % |  |
|  | bankfull_flow | flow at which the river begins to overlap the banks at a gauging station (obtained from stage-discharge relationships so may be derived by extrapolation) | m3 s-1 | UK National River Flow Archive using the NRFA API*, catchment timeseries of streamflow described in Table 2 and 3 |
|  | structurefull_flow | flow at which the river begins to overlap the wingwalls of a structure at a gauging station (obtained from stage-discharge relationships so may be derived by extrapolation) | m3 s-1 |  |
|  | max_gauging_flow_date | date and time when the maximum gauging flow was taken | - |  |
|  | max_gauging_flow | the maximum gauging flow - the highest manual measurement of flow taken at a gauging station | m3 s-1 |  |
|  | max_daily_flow | the maximum daily flow recorded in the daily flow time series times provided in CAMELS-GB v2 | m3 s-1 |  |
|  | daily_flow_extrap_dur | the percentage of time (excluding NaNs) that the daily flow timeseries is higher than the maximum gauging flow | % |  |
|  | max_hourly_flow | the maximum hourly flow recorded in the hourly flow time series times provided in CAMELS-GB v2 | m3 s-1 |  |
|  | hourly_flow_extrap_dur | the percentage of time (excluding NaNs) that the hourly flow timeseries is higher than the maximum gauging flow | % |  |
|  | qXX_uncert_upper | upper bound of the discharge uncertainty interval for the XX percentile flow given as a percentage of the XX percentile flow – estimates for XX values of 5, 25, 50, 75, 95, 99 are provided | % | Derived from Coxon et al (2015) |
|  | qXX_uncert_lower | lower bound of the discharge uncertainty interval for the XX percentile flow given as a percentage of the XX percentile flow – estimates for XX values of 5, 25, 50, 75, 95, 99 are provided | % |  |
|  | quncert_meta | metadata describing the reasons why discharge uncertainty estimates are (not) provided; Calculated discharge uncertainties; No stage-discharge measurements available; Less than 20 stage-discharge measurements available for most recent rating; Discharge uncertainty estimates not provided as the estimated uncertainty bounds were deemed to not accurately reflect the discharge uncertainty or because there was no sensible relationship between stage and discharge. | - |  |
|  | station_quality_pooling | indicates measurement suitability at the station, specifically that flows up to the 3rd highest annual maximum on record are likely to be within 30% of their true values (TRUE indicates the station is suitable for pooling, while FALSE indicates that it is not) | TRUE/ FALSE | UK National River Flow Archive (https://nrfa.ceh.ac.uk/data/about-data/peak-flow-data/indicative-suitabilities) |
|  | station_quality_qmed | indicates measurement suitability at the station, specifically that flows up to QMED (the median of the annual maximum flows or 1-in-2 year event) are likely to be within 30% of its true value (TRUE indicates the station is suitable for QMED, while FALSE indicates that it is not). | TRUE/ FALSE |  |
|  | station_quality_hourlyflow_issues | metadata describing if any station quality issues were found in the hourly flow timeseries including no data (no_data), truncated values for entire record (full_truncation) or specific parts of the timeseries (partial_truncation), continuity issues such as abrupt changes in flow regimes or prolonged gaps inconsistent with expected patterns for entire record (full_continuity) or specific parts of the timeseries (partial_continuity), very short periods of data availability prior to main period of record (partial_tracedata), and other issues (full_other_issues or partial_other_issues). | - | Fileni et al (2026) |
|  | station_quality_hourlyflow_comment | provides a detailed description of any station quality issues found in the hourly flow timeseries | - |  |
| Human Influence Attributes | benchmark_catch | benchmark catchment (Y indicates the catchment is part of the UK Benchmark Network, while N indicates that it is not) | Y/N | UK National River Flow Archive; Harrigan et al., (2018) |
|  | surfacewater_abs | mean surface water abstraction | mm day-1 | 1 km × 1 km abstractions for multiple purposes (csv file) and 1 km × 1 km discharges for multiple purposes (RACT netcdf file) (Rameshwaran et al., 2025) |
|  | groundwater_abs | mean groundwater abstraction | mm day-1 |  |
|  | discharges | mean discharges (daily discharges into water courses from water companies and other discharge permit holders reported to the Environment Agency) | mm day-1 |  |
|  | abs_agriculture_perc | percentage of total (groundwater and surface water) abstractions in catchment for agriculture | % |  |
|  | abs_amenities_perc | percentage of total (groundwater and surface water) abstractions in catchment for amenities | % |  |
|  | abs_energy_perc | percentage of total (groundwater and surface water) abstractions in catchment for energy production | % |  |
|  | abs_environmental_perc | percentage of total (groundwater and surface water) abstractions in catchment for environmental purposes | % |  |
|  | abs_industry_perc | percentage of total (groundwater and surface water) abstractions in catchment for industrial, commercial and public services | % |  |
|  | abs_watersupply_perc | percentage of total (groundwater and surface water) abstractions in catchment for water supply | % |  |
|  | abs_highloss_perc | percentage of total (groundwater and surface water) abstractions in catchment that is classified as ‘high loss’ | % |  |
|  | abs_mediumloss_perc | percentage of total (groundwater and surface water) abstractions in catchment that is classified as ‘medium loss’ | % |  |
|  | abs_lowloss_perc | percentage of total (groundwater and surface water) abstractions in catchment that is classified as ‘low loss’ | % |  |
|  | abs_verylowloss_perc | percentage of total (groundwater and surface water) abstractions in catchment that is classified as ‘very low loss’ | % |  |
|  | num_reservoir | number of reservoirs in the catchment | - | UK Reservoir Inventory (Durant and Counsell, 2018), SEPA’s publicly available controlled reservoirs register (http://map.sepa.org.uk/reservoirsfloodmap/Map.htm, last access: 2nd September, 2025) and Salwey et al., (2023) |
|  | reservoir_cap | total storage capacity of reservoirs in the catchment in megalitres | ML |  |
|  | reservoir_contributing_area | percentage of the overall catchment surface area that is drained through reservoirs | % |  |
|  | reservoir_normalised_upstream_capacity | ratio of the capacity of a reservoir to the average volume of precipitation received by the catchment in a year | - |  |
|  | reservoir_he | percentage of total reservoir storage in catchment used for hydroelectricty | % |  |
|  | reservoir_nav | percentage of total reservoir storage in catchment used for navigation | % |  |
|  | reservoir_drain | percentage of total reservoir storage in catchment used for drainage | % |  |
|  | reservoir_wr | percentage of total reservoir storage in catchment used for water resources | % |  |
|  | reservoir_fs | percentage of total reservoir storage in catchment used for flood storage | % |  |
|  | reservoir_env | percentage of total reservoir storage in catchment used for environmental | % |  |
|  | reservoir_nousedata | percentage of total reservoir storage in catchment where no use data were available | % |  |
|  | reservoir_year_first | year the first reservoir in the catchment was built | - |  |
|  | reservoir_year_last | year the last reservoir in the catchment was built | - |  |
| Groundwater Wells | gw_well_id | groundwater well identifier (corresponds to the ID provided by the British Geological Survey) | - | UK Hydrometric Register (Marsh and Hannaford, 2008) |
|  | gw_well_name | groundwater well name | - |  |
|  | gw_well_lat | groundwater well latitude | ° |  |
|  | gw_well_lon | groundwater well longitude | ° |  |
|  | gw_well_easting | groundwater well easting | m |  |
|  | gw_well_northing | groundwater well northing | m |  |
|  | gw_well_datum | the altitude of the point from which measurements are taken at a particular site | mAOD |  |
|  | gw_well_depth | depth of the groundwater well | m |  |
|  | aquifer | aquifer to which the water level variations in the wells are attributed | - |  |
|  | daily_gwlevel_period_start | first date that daily groundwater level series provided in CAMELS-GB v2 is available for this groundwater well | - | Groundwater level timeseries described in Table 4 |
|  | daily_gwlevel_period_end | end date that daily groundwater level series provided in CAMELS-GB v2 is available for this groundwater well | - |  |
|  | daily_gwlevel_perc_complete | percentage of days with groundwater level data | % |  |
|  | monthly_gwlevel_period_start | first date that monthly groundwater level series provided in CAMELS-GB v2 is available for this groundwater well | - |  |
|  | monthly_gwlevel_period_end | end date that monthly groundwater level series provided in CAMELS-GB v2 is available for this groundwater well | - |  |
|  | monthly_gwlevel_perc_complete | percentage of months with groundwater level data | % |  |

* https://nrfaapps.ceh.ac.uk/nrfa/nrfa-api.html, data downloaded on the 7th January 2025, last access to website 23rd September 2025

Table 2 Summary table of catchment daily hydro-meteorological timeseries available in CAMELS-GB v2

| Timeseries Class | Timeseries Name | Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Meteorological Timeseries (available from 1st October 1970 – 30th September 2022) | precipitation_cehgear | catchment averaged daily precipitation | mm day-1 | CEH-GEAR (Keller et al., 2015; Tanguy, 2021) |
|  | precipitation_haduk | catchment averaged daily precipitation | mm day-1 | HadUK-Grid (Hollis et al., 2019) |
|  | pet_chess | catchment averaged daily potential evapotranspiration for a well-watered grass (Penman-Monteith equation) | mm day-1 | CHESS-PE (Robinson et al., 2017, 2023b) |
|  | peti_chess | catchment averaged daily potential evapotranspiration for a well-watered grass (Penman-Monteith equation with a correction added for interception on days where rainfall has occurred) | mm day-1 |  |
|  | pet_hydrope | catchment averaged daily potential evapotranspiration for a well-watered grass (Penman-Monteith equation) | mm day-1 | Hydro-PE (Brown et al., 2023; Robinson et al., 2023c) |
|  | peti_hydrope | catchment averaged daily potential evapotranspiration for a well-watered grass (Penman-Monteith equation with a correction added for interception on days where rainfall has occurred) | mm day-1 |  |
|  | temperature_chess | catchment averaged daily temperature | °C | CHESS-met (Robinson et al., 2023a) |
|  | temperature_haduk | catchment averaged daily temperature | °C | HadUK-Grid (Hollis et al., 2019) |
| Hydrological Timeseries (available from 1st October 1970 – 30th September 2022) | discharge_spec | catchment specific discharge (converted to mm day-1 using catchment areas described in Section 2) | mm day-1 | UK National River Flow Archive using the NRFA API* |
|  | discharge_vol | catchment discharge | m3 s-1 |  |

* https://nrfaapps.ceh.ac.uk/nrfa/nrfa-api.html, data downloaded on the 7th January 2025, last access to website 23rd September 2025

Table 3 Summary table of catchment hourly hydro-meteorological timeseries available in CAMELS-GB v2

| Timeseries Class | Timeseries Name | Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Meteorological Timeseries (available from 1st October 1990 09:00 – 1st October 2022 08:00) | precipitation_cehgear | catchment hourly averaged precipitation from 1st October 1990 - 31st December 2019 | mm hour-1 | CEH-GEAR1hr (Lewis et al., 2018, 2022) |
|  | precipitation_gradgb | catchment hourly averaged precipitation from 1st January 2006 - 31st December 2023 | mm hour-1 | Gauge-Radar Precipitation Dataset (1 hour and 1 km) for Great Britain, GRaD-GB(1H1K) (Qiu et al., 2025b, a) |
| Hydrological Timeseries (available from 1st October 1990 09:00 – 1st October 2022 08:00) | discharge_spec | catchment specific discharge (converted to mm hour-1 using catchment areas described in Section 2) | mm hour-1 | The flows and level data were obtained from SEPA via the timeseries data service (https://timeseriesdoc.sepa.org.uk/; last access 23rd September 2025), from EA primarily through the Hydrology Data Explorer (https://environment.data.gov.uk/hydrology; last access 23rd September 2025) and, where unavailable, with staff assistance, and from NRW entirely with staff assistance. Flags derived from UKFlows15. |
|  | discharge_vol | catchment discharge | m3 s-1 |  |
|  | discharge_flag | numeric flag that indicates the quality of the flow data (full description of flags can be found in Figure S5 and Table S1-S3) | - |  |
|  | level | height of the river (measured in metres above river bed) | m |  |
|  | level_flag | numeric flag that indicates the quality of the level data (full description of flags can be found in Figure S5 and Table S1-S3) | - |  |

Table 4. Summary table of daily and monthly groundwater level timeseries available in CAMELS-GB v2

| Timeseries Class | Timeseries Name | Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Groundwater Level Timeseries (daily and monthly timeseries for variable time periods) | groundwater_level | groundwater level for a specific borehole at either daily or monthly timesteps | mAOD | British Geological Survey (BGS), National Groundwater Level Archive [online]. Available at: https://www2.bgs.ac.uk/groundwater/datainfo/levels/ngla.html (Accessed: 01 Feb 2025). |

References

BGS hydrogeology 625k: https://www.bgs.ac.uk/products/hydrogeology/maps.html, last access: 8 October 2019.

Brown, M. J., Robinson, E. L., Kay, A. L., Chapman, R., Bell, V. A., and Blyth, E. M.: Potential evapotranspiration derived from HadUK-Grid 1km gridded climate observations 1969-2022 (Hydro-PE HadUK-Grid), 2023.

Durant, M. J. and Counsell, C. J.: Inventory of reservoirs amounting to 90% of total UK storage, NERC Environmental Information Data Centre, https://doi.org/10.5285/f5a7d56c-cea0-4f00-b159-c3788a3b2b38, 2018.

Harrigan, S., Hannaford, J., Muchan, K., and Marsh, T. J.: Designation and trend analysis of the updated UK Benchmark Network of river flow stations: the UKBN2 dataset, Hydrology Research, 49, 552–567, https://doi.org/10.2166/nh.2017.058, 2018.

Hiederer, R.: Mapping Soil Properties for Europe- Spatial Representation of Soil Database Attributes, Luxembourg, 2013a.

Hiederer, R.: Mapping soil typologies: spatial decision support applied to the European Soil Database., Publications Office, Luxembourg, 2013b.

Hollis, D., McCarthy, M., Kendon, M., Legg, T., and Simpson, I.: HadUK-Grid—A new UK dataset of gridded climate observations, Geoscience Data Journal, 6, 151–159, https://doi.org/10.1002/gdj3.78, 2019.

Keller, V. D. J., Tanguy, M., Prosdocimi, I., Terry, J. A., Hitt, O., Cole, S. J., Fry, M., Morris, D. G., and Dixon, H.: CEH-GEAR: 1 km resolution daily and monthly areal rainfall estimates for the UK for hydrological and other applications, Earth System Science Data, 7, 143–155, https://doi.org/10.5194/essd-7-143-2015, 2015.

Ladson, A. R., Brown, R., Neal, B., and Nathan, R.: A Standard Approach to Baseflow Separation Using The Lyne and Hollick Filter, Australasian Journal of Water Resources, 17, 25–34, https://doi.org/10.7158/13241583.2013.11465417, 2013.

Lewis, E., Quinn, N., Blenkinsop, S., Fowler, H. J., Freer, J., Tanguy, M., Hitt, O., Coxon, G., Bates, P., and Woods, R.: A rule based quality control method for hourly rainfall data and a 1 km resolution gridded hourly rainfall dataset for Great Britain: CEH-GEAR1hr, Journal of Hydrology, 564, 930–943, https://doi.org/10.1016/j.jhydrol.2018.07.034, 2018.

Lewis, E., Quinn, N., Blenkinsop, S., Fowler, H. J., Freer, J., Tanguy, M., Hitt, O., Coxon, G., Bates, P., Woods, R., Fry, M., Chevuturi, A., Swain, O., and White, S. M.: Gridded estimates of hourly areal rainfall for Great Britain 1990-2016 [CEH-GEAR1hr] v2, 2022.

Marsh, T. J. and Hannaford, J.: UK Hydrometric Register, Centre for Ecology and Hydrology, Wallingford, UK, 2008.

Marston, C., Rowland, C. S., O&#39;Neil, A. W., and Morton, R. D.: Land Cover Map 2021 (25m rasterised land parcels, GB), , https://doi.org/10.5285/a1f85307-cad7-4e32-a445-84410efdfa70, 2022.

Marston, C. G., Morton, R. D., O&#39;Neil, A. W., and Rowland, C. S.: Land Cover Map 2022 (25m rasterised land parcels, GB), , https://doi.org/10.5285/c9449bf5-b8f6-4a1c-b3eb-0d70575cba39, 2024.

Morris, D. G. and Flavin, R. W.: A digital terrain model for hydrology, in: Proc 4th International Symposium on Spatial Data Handling, Zürich, 250–262, 1990.

Morton, R. D., Marston, C. G., O&#39;Neil, A. W., and Rowland, C. S.: Land Cover Map 2017 (25m rasterised land parcels, GB), , https://doi.org/10.5285/499212cd-d64a-43ba-b801-95402e4d4098, 2020a.

Morton, R. D., Marston, C. G., O&#39;Neil, A. W., and Rowland, C. S.: Land Cover Map 2018 (25m rasterised land parcels, GB), , https://doi.org/10.5285/25c6451b-5c88-40da-9a63-c3ec473e4874, 2020b.

Morton, R. D., Marston, C. G., O&#39;Neil, A. W., and Rowland, C. S.: Land Cover Map 2019 (25m rasterised land parcels, GB), , https://doi.org/10.5285/f15289da-6424-4a5e-bd92-48c4d9c830cc, 2020c.

Morton, R. D., Marston, C. G., O&#39;Neil, A. W., and Rowland, C. S.: Land Cover Map 2020 (25m rasterised land parcels, GB), , https://doi.org/10.5285/6c22cf6e-b224-414e-aa85-900325baedbd, 2021.

Pelletier, J. D., Broxton, P. D., Hazenberg, P., Zeng, X., Troch, P. A., Niu, G., Williams, Z. C., Brunke, M. A., and Gochis, D.: Global 1-km Gridded Thickness of Soil, Regolith, and Sedimentary Deposit Layers, ORNL DAAC, https://doi.org/10.3334/ORNLDAAC/1304, 2016.

Qiu, X., Green, A. C., Blenkinsop, S., and Fowler, H. J.: Developing a Radar-Rain Gauge Hourly Blended Precipitation Dataset for Great Britain Using the Gauss Blending Method, https://doi.org/10.2139/ssrn.5364302, 24 July 2025a.

Qiu, X., Green, A. C., Blenkinsop, S., and Fowler, H. J.: Quality Control of the Composite Radar Quantitative Precipitation Estimation Product for Great Britain, https://doi.org/10.2139/ssrn.5215346, 13 April 2025b.

Rameshwaran, P., Bell, V. A., Davies, H. N., Sadler, P., Beverton, A., Thornton, R., and Rhodes-Smith, M.: Gridded actual groundwater, surface water and tidal water abstraction, discharge and Hands-off Flow datasets for England (1999 to 2014), https://doi.org/10.5285/18886F95BA84447F997EFAC96DF456AD, 2025.

Robinson, E. L., Blyth, E. M., Clark, D. B., Finch, J., and Rudd, A. C.: Trends in atmospheric evaporative demand in Great Britain using high-resolution meteorological data, Hydrology and Earth System Sciences, 21, 1189–1224, https://doi.org/10.5194/hess-21-1189-2017, 2017.

Robinson, E. L., Blyth, E. M., Clark, D. B., Comyn-Platt, E., Rudd, A. C., and Wiggins, M.: Climate hydrology and ecology research support system meteorology dataset for Great Britain (1961-2019) [CHESS-met], 2023a.

Robinson, E. L., Blyth, E. M., Clark, D. B., Comyn-Platt, E., Rudd, A. C., and Wiggins, M.: Climate hydrology and ecology research support system potential evapotranspiration dataset for Great Britain (1961-2019) [CHESS-PE], 2023b.

Robinson, E. L., Brown, M. J., Kay, A. L., Lane, R. A., Chapman, R., Bell, V. A., and Blyth, E. M.: Hydro-PE: gridded datasets of historical and future Penman–Monteith potential evaporation for the United Kingdom, Earth System Science Data, 15, 4433–4461, https://doi.org/10.5194/essd-15-4433-2023, 2023c.

Rowland, C. S., Morton, R. D., Carrasco, L., McShane, G., O&#39;Neil, A. W., and Wood, C. M.: Land Cover Map 2015 (25m raster, GB), , https://doi.org/10.5285/bb15e200-9349-403c-bda9-b430093807c7, 2017.

Rowland, C. S., Marston, C. G., Morton, R. D., and O&#39;Neil, A. W.: Land Cover Map 1990 (25m raster, GB) v2, , https://doi.org/10.5285/1be1912a-916e-42c0-98cc-16460fac00e8, 2020.

Salwey, S., Coxon, G., Pianosi, F., Singer, M. B., and Hutton, C.: National-Scale Detection of Reservoir Impacts Through Hydrological Signatures, Water Resources Research, 59, e2022WR033893, https://doi.org/10.1029/2022WR033893, 2023.

Tanguy, V. D. J., M. ;Dixon, H. ;Prosdocimi, I. ;Morris, D. G. ;Keller: Gridded estimates of daily and monthly areal rainfall for the United Kingdom (1890-2019) [CEH-GEAR], , https://doi.org/10.5285/dbf13dd5-90cd-457a-a986-f2f9dd97e93c, 2021.